SIMATS ENGINEERING

ASSESSMENT TOOL 3

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Software:Cisco Packet Tracer,Wireshark

Annexure A

Debugging & Optimisation Task

1. A network administrator has configured a workstation with the following settings:

- IP Address: 192.168.10.65

- Subnet Mask: 255.255.255.192 (/26)

- Default Gateway: 192.168.10.1

However, the workstation is unable to communicate with other systems configured in the subnet 192.168.10.0/26.

Task

Debug the network configuration by:

- Identifying the subnet to which the configured IP address belongs.

- Determining the network address, broadcast address, and valid host range.

- Verifying whether the configured IP address and gateway are valid.

- Identifying the configuration errors.

- Optimizing the configuration by assigning an appropriate IP address and default gateway.

- Calculating the total number of usable host addresses after correction.

2. Two devices are configured with the following IPv6 addresses:

- Device A: 2001:db8::ff00:42:8329/64

- Device B: 2001:db8:0:0:1::1/64

Although both devices are connected to the same network, communication fails.

### Task

Debug the IPv6 configuration by:

- Expanding both IPv6 addresses into their complete notation.

- Determining the network prefix of each device.

- Verifying whether both devices belong to the same subnet.

- Identifying the addressing or prefix mismatch.

- Optimizing the IPv6 configuration by assigning appropriate addresses.

- Calculating the number of available host addresses within the corrected subnet.

3. An organization has allocated the network 192.168.1.0/24 into several departments using the subnet masks /26, /27, and /28. Some departments report insufficient IP addresses, while others have a large number of unused addresses.

Task

Analyze and optimize the subnet allocation by:

- Identifying the network address, broadcast address, and host range for each subnet.

- Calculating the number of usable and unused IP addresses.

- Identifying inefficient subnet allocations.

- Debugging the addressing plan by locating subnet allocation mistakes.

- Recommending an optimized subnetting scheme using VLSM to improve address utilization while supporting current departmental requirements.

4. Two devices are configured with the following IPv6 addresses:

- Device A: 2001:db8::ff00:42:8329/64

- Device B: 2001:db8:0:0:1::1/64

Although both devices are connected to the same network, communication fails.

Task

Debug the IPv6 configuration by:

- Expanding both IPv6 addresses into their complete notation.

- Determining the network prefix of each device.

- Verifying whether both devices belong to the same subnet.

- Identifying the addressing or prefix mismatch.

- Optimizing the IPv6 configuration by assigning appropriate addresses.

- Calculating the number of available host addresses within the corrected subnet.

5. A router contains the following routing table.

| Destination Network | Next Hop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| Default Route | 10.0.0.1 |

Users report that packets destined for 192.168.2.100 are not reaching the intended network.

Task

Debug the routing configuration by:

- Identifying the matching routing entry using the longest-prefix match.

- Determining the next-hop router selected for packet forwarding.

- Verifying whether the destination IP belongs to the intended subnet.

- Identifying any routing configuration errors.

- Optimizing the routing table to improve packet forwarding efficiency.

- Identifying the route that will be used if no matching destination exists.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Identification | 20% | Accurately identifies all errors and root causes with clear understanding | Identifies most errors with minor gaps | Identifies some errors but misses key issues | Unable to identify errors correctly |
| Debugging | 25% | Applies systematic debugging techniques effectively to resolve issues | Uses appropriate debugging methods with minor errors | Limited debugging approach; requires guidance | Unable to debug or resolve issues |
| Optimization | 20% | Optimizes code by significantly improving time complexity using efficient algorithms | Improves time complexity with minor gaps in efficiency | Limited improvement in time complexity | No improvement; inefficient approach retained |
| Analysis | 20% | Demonstrates strong logical reasoning and clear justification of solutions | Shows reasonable analysis with some justification | Limited analysis; weak reasoning | No analytical reasoning demonstrated |
| Code Quality | 15% | Produces clean, well-structured code with proper comments and documentation | Code is mostly clear with minor documentation gaps | Code lacks structure and proper documentation | Poorly written code with no documentation |

# 1. Debugging IPv4 Configuration

### Given Configuration

- IP Address = 192.168.10.65

- Subnet Mask = 255.255.255.192 (/26)

- Default Gateway = 192.168.10.1

## Step 1: Identify the Subnet

Subnet Mask = /26

Block Size:

256−192=64256-192=64256−192=64

Subnets are:

- 192.168.10.0 – 192.168.10.63

- 192.168.10.64 – 192.168.10.127

- 192.168.10.128 – 192.168.10.191

- 192.168.10.192 – 192.168.10.255

Since 192.168.10.65 falls between 64–127, it belongs to:

Subnet: 192.168.10.64/26

## Step 2: Network Details

- Network Address: 192.168.10.64

- Broadcast Address: 192.168.10.127

- Valid Host Range: 192.168.10.65 – 192.168.10.126

## Step 3: Verify IP Address

IP = 192.168.10.65

✔ Valid Host Address

## Step 4: Verify Default Gateway

Gateway = 192.168.10.1

192.168.10.1 belongs to subnet

192.168.10.0/26

The workstation belongs to

192.168.10.64/26

❌ Gateway is in a different subnet.

## Step 5: Configuration Error

Error: Incorrect Default Gateway.

## Step 6: Optimized Configuration

| Parameter | Correct Value |
| --- | --- |
| IP Address | 192.168.10.65 |
| Subnet Mask | 255.255.255.192 |
| Default Gateway | 192.168.10.65 (if router interface uses this IP, otherwise use another valid router IP in the subnet such as 192.168.10.126 or 192.168.10.66) |

In practice, the router interface should use a valid address within 192.168.10.64/26 (e.g., 192.168.10.126), not the host's own IP.

## Step 7: Usable Hosts

26−2=622^{6}-2=6226−2=62

Usable Hosts = 62

# 2. IPv6 Debugging

### Given

Device A

2001:db8::ff00:42:8329/64

Device B

2001:db8:0:0:1::1/64

## Expanded Addresses

### Device A

2001:0db8:0000:0000:0000:ff00:0042:8329

### Device B

2001:0db8:0000:0000:0001:0000:0000:0001

## Network Prefix

### Device A

2001:db8:0:0::/64

### Device B

2001:db8:0:0::/64

## Same Subnet?

Yes.

Both have identical first 64 bits.

✔ Same subnet.

## Why Communication Fails

Possible reasons:

- Incorrect default gateway

- IPv6 disabled

- Interface down

- Firewall blocking ICMPv6

- Duplicate Address Detection failure

No addressing mismatch exists.

## Optimized Configuration

Device A

2001:db8:0:0::10/64

Device B

2001:db8:0:0::20/64

Default Gateway

2001:db8:0:0::1

## Available Hosts

Host Bits

64

2642^{64}264

= 18,446,744,073,709,551,616 Hosts

# 3. Optimizing Subnet Allocation (VLSM)

### Given Network

192.168.1.0/24

Current allocations:

- /26

- /27

- /28

## Existing Subnets

### /26

Network

192.168.1.0

Broadcast

192.168.1.63

Host Range

192.168.1.1 – 192.168.1.62

Usable Hosts

62

### /27

Network

192.168.1.64

Broadcast

192.168.1.95

Host Range

192.168.1.65 – 192.168.1.94

Usable Hosts

30

### /28

Network

192.168.1.96

Broadcast

192.168.1.111

Host Range

192.168.1.97 – 192.168.1.110

Usable Hosts

14

## Unused Addresses

Remaining addresses

192.168.1.112 – 192.168.1.255

## Problems Identified

- Small departments may waste addresses with /26.

- Large departments may not have enough addresses with /28.

- Address utilization is inefficient.

## Optimized Using VLSM

Allocate largest departments first:

| Department Size | Suggested Prefix | Usable Hosts |
| --- | --- | --- |
| Large | /26 | 62 |
| Medium | /27 | 30 |
| Small | /28 | 14 |
| Very Small | /29 | 6 |

Benefits:

- Better utilization

- Less address wastage

- Easy future expansion

# 4. IPv6 Debugging (Same as Question 2)

### Expanded Address

Device A

2001:0db8:0000:0000:0000:ff00:0042:8329

Device B

2001:0db8:0000:0000:0001:0000:0000:0001

Network Prefix

2001:db8:0:0::/64

Both belong to the same subnet.

Optimized addresses:

2001:db8:0:0::10/64

2001:db8:0:0::20/64

Gateway

2001:db8:0:0::1

Available Hosts

2⁶⁴

# 5. Debugging Router Configuration

### Routing Table

| Destination | Next Hop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| Default | 10.0.0.1 |

Destination IP

192.168.2.100

## Longest Prefix Match

Destination belongs to

192.168.2.0/24

Matching Route

192.168.2.0/24

## Next Hop

10.0.0.3

## Destination Verification

Destination

192.168.2.100

Host Range

192.168.2.1 – 192.168.2.254

✔ Valid Host

## Possible Routing Errors

- Incorrect next-hop IP

- Interface down

- Missing route

- Incorrect subnet mask

- ACL/firewall blocking packets

## Optimized Routing Table

| Destination | Next Hop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| Default (0.0.0.0/0) | 10.0.0.1 |

This ensures correct packet forwarding and provides a fallback route for unknown destinations.

## Default Route

If no route matches:

0.0.0.0/0

Packet is forwarded to:

10.0.0.1