SIMATS ENGINEERING

ASSESSMENT TOOL 2

Case study

COURSE NAME: Computer Networks			COURSE CODE:	CSA07

COURSE OUTCOME COVERED

CO3: Analyze the performance of core network protocols such as TCP, UDP, HTTP, and DNS under varying conditions

Assessment Tool Weightage: 40%

Case study

QUESTIONS:4						                   Total Marks:40

1.A multinational IT company uses a cloud-based video conferencing platform for daily client meetings. During peak business hours, employees experience frequent voice distortion, frozen video frames, and delayed communication. Network monitoring reports indicate an 8% packet loss, 180 ms jitter, and fluctuating bandwidth utilization. The IT administrator suspects that the issue is related to the transport protocol or application-level buffering strategy.

### Questions

- Analyze whether the problem is primarily caused by the transport protocol (TCP/UDP) or the application layer.

- Justify your answer based on the communication requirements of real-time multimedia applications.

- Recommend suitable protocol-level or application-level improvements to enhance conferencing quality.

2.A multinational software company hosts its web services across multiple continents. Employees frequently report long delays before the application homepage loads. Network analysis shows that the average DNS lookup time is approximately 800 ms, significantly affecting user experience. The organization plans to redesign its DNS infrastructure while ensuring continuous service availability during server failures.

### Questions

- Analyze the reasons behind the high DNS resolution time.

- Propose a suitable DNS architecture using techniques such as Anycast DNS, DNS pre-fetching, and TTL optimization to reduce the lookup time below 50 ms.

- Evaluate the advantages and limitations of your proposed architecture in terms of performance, scalability, and availability.

3.A cloud service provider delivers applications to millions of users worldwide. During promotional events, users experience intermittent delays in accessing cloud-hosted services. Investigation reveals that DNS queries are taking nearly 800 ms because all requests are directed to a single primary DNS server. The company wants a highly available DNS solution capable of handling global traffic efficiently.

### Questions

- Examine the impact of centralized DNS architecture on application performance.

- Design a distributed DNS solution that minimizes lookup latency while maintaining high availability.

- Analyze how TTL values, Anycast routing, and DNS caching influence system performance and fault tolerance.

4.A financial institution operates an electronic stock trading platform where thousands of buy and sell orders are submitted every second. During market opening hours, the average order acknowledgment latency increases from 1 ms to 40 ms, resulting in delayed trade confirmations and customer dissatisfaction. Network engineers observe increased TCP retransmissions, longer connection establishment times, and excessive buffering.

### Questions

- Analyze three possible TCP-related factors responsible for the latency increase, considering flow control, Nagle's algorithm, and SYN queue limitations.

- Explain how each factor affects transaction processing in high-frequency systems

- Recommend appropriate TCP configuration changes to reduce latency and improve transaction reliability.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Case | 25 | Thorough understanding; clearly identifies all key issues | Good understanding with most issues identified | Basic understanding; some issues missed | Poor understanding; problem not identified correctly |
| Application of Concepts | 25 | Effectively applies relevant concepts to analyze the case deeply | Applies appropriate concepts with minor gaps | Limited application of concepts | Fails to apply relevant concepts |
| Analysis | 20 | In-depth analysis with strong reasoning and insights | Good analysis with logical reasoning | Basic analysis with limited depth | Weak or no analysis; lacks reasoning |
| Solution Design | 20 | Innovative, feasible solutions with strong justification | Logical solutions with adequate justification | Basic solutions with weak justification | Incorrect or no solution provided |
| Clarity | 10 | Clear, well-structured, and easy to understand | Organized and understandable presentation | Limited clarity and structure | Poor clarity; difficult to understand |

# Question 1

### Case

A multinational IT company uses a cloud-based video conferencing platform. During peak hours, users experience:

- 8% packet loss

- 180 ms jitter

- Voice distortion

- Frozen video

- Delayed communication

## 1. Analyze the Cause

The primary issue is caused by both the transport protocol (UDP) and the application layer, with the network conditions being the root cause.

- Real-time video conferencing typically uses UDP because it does not retransmit lost packets, reducing delay.

- Packet loss (8%) and high jitter (180 ms) degrade audio and video quality.

- The application's jitter buffer may be too small or poorly configured to handle delay variation.

## 2. Justification

Real-time multimedia applications require:

- Low latency

- Low jitter

- Continuous playback

UDP is preferred because:

- No connection setup delay.

- No retransmission delays.

- Suitable for voice and video where timely delivery is more important than perfect accuracy.

TCP is unsuitable because:

- Retransmits lost packets.

- Causes additional delay.

- Leads to buffering and frozen video.

Thus, the communication issues are mainly due to poor network quality and insufficient application buffering rather than UDP itself.

## 3. Recommendations

### Protocol-Level Improvements

- Use UDP with RTP (Real-time Transport Protocol).

- Enable Forward Error Correction (FEC).

- Implement QoS (Quality of Service) to prioritize voice/video traffic.

- Use adaptive bitrate streaming.

### Application-Level Improvements

- Increase adaptive jitter buffer size.

- Implement packet-loss concealment.

- Dynamically adjust video quality based on bandwidth.

- Reduce video resolution during congestion.

## Conclusion

Using UDP with RTP, adaptive buffering, QoS, and FEC minimizes latency, improves voice quality, and provides smoother video conferencing.

# Question 2

### Case

Global web services experience:

- DNS lookup time = 800 ms

- Slow homepage loading

Target:

Reduce lookup time below 50 ms.

## 1. Reasons for High DNS Resolution Time

Possible causes include:

- Single centralized DNS server

- Long network distance

- DNS server overload

- Cache misses

- High TTL expiration frequency

- Lack of regional DNS servers

## 2. Proposed DNS Architecture

### (a) Anycast DNS

Deploy DNS servers worldwide using Anycast routing.

Benefits:

- Users connect to the nearest DNS server.

- Lower latency.

- Automatic failover.

### (b) DNS Pre-fetching

Browsers resolve frequently accessed domains before users request them.

Benefits:

- Reduces waiting time.

- Faster webpage loading.

### (c) TTL Optimization

Use moderate TTL values (300–600 seconds).

Benefits:

- Cached records remain available.

- Faster repeated queries.

- Allows timely updates.

### Proposed Architecture

Client

│

Nearest Anycast DNS Server

│

Regional DNS Cache

│

Authoritative DNS Servers

Expected DNS lookup:

< 50 ms

## 3. Evaluation

### Advantages

- Lower lookup latency.

- Better scalability.

- High availability.

- Automatic load balancing.

- Faster disaster recovery.

### Limitations

- Higher deployment cost.

- Complex DNS management.

- Cache consistency challenges.

- Requires global infrastructure.

## Conclusion

Anycast DNS with caching and optimized TTL significantly improves DNS performance while ensuring continuous service availability.

# Question 3

### Case

Cloud provider:

- Single DNS server

- DNS lookup = 800 ms

Goal:

Highly available distributed DNS.

## 1. Impact of Centralized DNS

Problems:

- Single point of failure.

- High latency.

- Server overload.

- Poor scalability.

- Longer application response time.

Result:

Slow cloud services and poor user experience.

## 2. Distributed DNS Design

Users

│

Internet

│

Anycast DNS

┌────┼─────┐

Asia Europe America

│      │      │

Regional DNS Servers

│      │      │

Authoritative DNS

Features:

- Multiple global DNS servers.

- Automatic nearest-server selection.

- Load balancing.

- Redundancy.

## 3. Influence of DNS Techniques

### TTL

Small TTL

- Faster updates.

- More DNS queries.

Large TTL

- Better caching.

- Lower DNS traffic.

Recommended:

300–600 seconds.

### Anycast Routing

Advantages:

- Lowest latency.

- Automatic failover.

- Better traffic distribution.

### DNS Caching

Advantages:

- Faster responses.

- Reduced DNS traffic.

- Lower server load.

## Conclusion

Distributed DNS using Anycast, caching, and optimized TTL improves latency, scalability, and fault tolerance.

# Question 4

### Case

Electronic trading platform:

Latency increases:

1 ms → 40 ms

Observations:

- TCP retransmissions

- Long connection setup

- Excessive buffering

## 1. Three TCP Factors

### (a) Flow Control

TCP receiver advertises a receive window.

If the receiver cannot process data quickly:

- Sender slows transmission.

- Queue builds up.

- Latency increases.

### (b) Nagle's Algorithm

Nagle combines multiple small packets into one.

Benefits:

- Reduces network overhead.

Problem:

- Delays transmission of small trading messages.

- Increases acknowledgment latency.

For stock trading:

Every millisecond matters.

### (c) SYN Queue Limitations

During peak load:

- Connection requests exceed SYN queue size.

- New TCP connections wait or are dropped.

- Clients retransmit SYN packets.

- Connection setup delay increases.

## 2. Effect on High-Frequency Trading

| TCP Factor | Effect |
| --- | --- |
| Flow Control | Slows order transmission when receiver buffers fill. |
| Nagle's Algorithm | Delays small order packets, increasing acknowledgment time. |
| SYN Queue Overflow | Delays or drops new client connections during peak trading. |

## 3. Recommended TCP Configuration

### Disable Nagle's Algorithm

Enable:

TCP_NODELAY

Benefit:

Immediate transmission of trading orders.

### Increase SYN Queue Size

Benefits:

- Handles more simultaneous connections.

- Reduces dropped SYN requests.

- Faster connection establishment.

### Increase Receive Buffer

Benefits:

- Reduces congestion.

- Improves throughput.

### Enable TCP Window Scaling

Benefits:

- Better utilization on high-speed links.

- Reduced bottlenecks.

### Use Persistent Connections

Benefits:

- Avoid repeated TCP handshakes.

- Lower latency.

## Conclusion

Disabling Nagle's algorithm, increasing SYN queue capacity, optimizing TCP buffers, enabling window scaling, and using persistent TCP connections significantly reduce latency and improve the reliability of high-frequency stock trading systems.

# Overall Summary

| Question | Key Solution |
| --- | --- |
| Q1 | Use UDP + RTP, QoS, adaptive jitter buffer, and Forward Error Correction (FEC) for high-quality real-time conferencing. |
| Q2 | Implement Anycast DNS, DNS pre-fetching, caching, and optimized TTL (300–600 s) to reduce lookup time below 50 ms. |
| Q3 | Replace centralized DNS with a distributed Anycast DNS architecture using regional servers and DNS caching for low latency and high availability. |
| Q4 | Reduce TCP latency by disabling Nagle's algorithm (TCP_NODELAY), increasing the SYN queue, enabling TCP window scaling, enlarging receive buffers, and using persistent connections. |