SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME:	CSA07 COURSE CODE: Computer Networks

COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across networks. (BL2, BL3)

Assessment Tool Weightage: 30%

Annexure A

CONCEPT MAPPING

1. A partially completed concept map is given below:

Application → Presentation → ______ → Transport → Network → ______ → Physical.

Analyze the missing layers and explain how their functions are essential for reliable communication.

2. Construct a concept map linking data units (Segment, Packet, Frame, Bits) with their respective OSI layers and analyze how transformation of data units supports transmission.

3.Given the concept map:

DataLink→MACAddress→LocalDelivery
Network → IP Address → Global Delivery

Explain how these relationships ensure complete data delivery across networks.

4. A concept map shows:

ErrorControl
→DataLinkLayer→ErrorDetection
→ Transport Layer → Error Correction

Analyze how the interaction between these layers improves reliability in data transmission.

5. Develop a concept map for web browsing using OSI layers and analyze how failure in one layer affects the entire communication process.

6. A network issue is represented in the concept map:

Physical ✔ → Data Link ✔ → Network ✖ → Transport ✖ → Application ✖

Analyze the problem and explain how the concept map helps in troubleshooting.

RUBRICS FOR CALCULATION

| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Concept Identification | 20% | Accurately identifies all relevant OSI layers, concepts, and relationships | Identifies most concepts with minor omissions | Identifies limited concepts; some inaccuracies | Unable to identify key concepts correctly |
| Concept Mapping Structure | 25% | Constructs a clear, logical, and well-organized concept map with proper hierarchy and connections | Concept map is mostly clear with minor structural issues | Concept map lacks clarity, organization, or proper flow | No clear structure or incorrect mapping |
| Linking & Relationships | 20% | Clearly establishes correct relationships between layers, functions, and processes | Relationships are mostly correct with minor errors | Limited or partially correct relationships | Incorrect or missing relationships |
| Application & Analysis | 20% | Effectively applies concepts to explain processes (e.g., encapsulation, error control) with strong reasoning | The application is correct but lacks depth in analysis | Basic application with weak explanation | Unable to apply concepts correctly |
| Explanation & Clarity | 15% | Provides a clear, concise, and well-structured explanation supporting the concept map | Explanation is understandable with minor gaps | Explanation is unclear or incomplete | No clear or meaningful explanation |

# 1. Missing OSI Layers

### Concept Map

Application

↓

Presentation

↓

Session

↓

Transport

↓

Network

↓

Data Link

↓

Physical

### Analysis

The missing layers are:

- Session Layer

  - Establishes, manages, and terminates communication sessions.

  - Provides synchronization and recovery during interruptions.

  - Ensures continuous communication between applications.

- Data Link Layer

  - Converts packets into frames.

  - Performs error detection using CRC.

  - Controls access to the physical medium using MAC addresses.

Both layers improve reliable communication by maintaining sessions, detecting transmission errors, and ensuring correct frame delivery.

# 2. Concept Map: Data Units and OSI Layers

### Concept Map

Application

↓

Presentation

↓

Session

↓

Transport

│

Segment

↓

Network

│

Packet

↓

Data Link

│

Frame

↓

Physical

│

Bits

### Analysis

As data moves through the OSI model, it undergoes encapsulation:

- Transport Layer → Segment

- Network Layer → Packet

- Data Link Layer → Frame

- Physical Layer → Bits

Each layer adds its own header (and trailer at Data Link), allowing addressing, routing, error detection, and reliable transmission. At the receiver, decapsulation removes these headers to reconstruct the original data.

# 3. MAC Address and IP Address Relationship

### Concept Map

Data Link Layer

↓

MAC Address

↓

Local Delivery

Network Layer

↓

IP Address

↓

Global Delivery

### Analysis

- MAC Address identifies devices within the same local network and ensures local frame delivery.

- IP Address identifies devices across different networks and enables routing over the Internet.

Together, the IP address determines the destination network, while the MAC address delivers the frame to the correct device within that network, ensuring complete end-to-end communication.

# 4. Error Control Concept Map

### Concept Map

Error Control

│

┌────┴────┐

│         │

Data Link  Transport

Layer      Layer

│           │

Error      Error

Detection Correction

### Analysis

- Data Link Layer

  - Detects transmission errors using CRC or checksum.

  - Requests retransmission of corrupted frames.

- Transport Layer

  - Uses acknowledgments, sequence numbers, and retransmissions.

  - Ensures reliable end-to-end delivery.

Together, these layers provide reliable communication by detecting errors locally and correcting lost or corrupted data across the entire network.

# 5. Web Browsing Using OSI Layers

### Concept Map

Web Browser

↓

Application (HTTP/HTTPS)

↓

Presentation (Encryption, SSL/TLS)

↓

Session (Session Management)

↓

Transport (TCP)

↓

Network (IP Routing)

↓

Data Link (Frames, MAC)

↓

Physical (Signals)

### Analysis

When a user opens a website:

- Application Layer sends HTTP/HTTPS requests.

- Presentation Layer encrypts data using SSL/TLS.

- Session Layer maintains the browsing session.

- Transport Layer guarantees reliable delivery through TCP.

- Network Layer routes packets to the destination.

- Data Link Layer frames the packets and uses MAC addresses.

- Physical Layer transmits bits over the communication medium.

If any layer fails, communication is interrupted. For example:

- Transport failure → webpage cannot load.

- Network failure → destination cannot be reached.

- Physical failure → no network connectivity.

Thus, all OSI layers must function together for successful web browsing.

# 6. Network Troubleshooting

### Concept Map

Physical ✔

↓

Data Link ✔

↓

Network ✖

↓

Transport ✖

↓

Application ✖

### Analysis

The Physical and Data Link layers are functioning correctly, indicating that cables, switches, and local communication are operational.

The failure begins at the Network Layer, which may be caused by:

- Incorrect IP configuration

- Router failure

- Routing table errors

- Network congestion

Since the Network Layer fails, the Transport Layer cannot establish reliable communication, and the Application Layer cannot access services such as web browsing or email.

The concept map helps identify the exact layer where communication fails, making troubleshooting faster and more systematic.

# Conclusion

The OSI model divides communication into seven layers, each performing specific functions. Concept maps clearly illustrate how these layers interact through encapsulation, addressing, routing, error control, and troubleshooting. Understanding these relationships enables reliable data transmission and simplifies network problem diagnosis.