SIMATS ENGINEERING

ASSESSMENT TOOL 1

Experiments

COURSE NAME: Computer Networks			COURSE CODE:	CSA07

COURSE OUTCOME COVERED

CO3: Analyze the performance of core network protocols such as TCP, UDP, HTTP, and DNS under varying conditions

Assessment Tool Weightage: 40%

SOFTWARE: CISCO PACKET TRACER, WIRESHARK

EXPERIMENTS

QUESTIONS:4						                Total Marks:40

- Design a simple LAN consisting of two PCs connected through a switch using Cisco Packet Tracer. Configure appropriate IP addresses for both hosts and verify connectivity using the ping command. Simultaneously capture the packet exchange using Wireshark. Identify the Ethernet frame fields such as Source MAC Address, Destination MAC Address, EtherType, and Frame Check Sequence (FCS). Explain how the Data Link layer uses MAC addressing to ensure successful frame delivery within the local network.

- Create a network topology consisting of two LANs connected through a router in Cisco Packet Tracer. Configure IPv4 addressing, subnet masks, and default gateways for all devices, and verify end-to-end communication using the ping command. Capture the ICMP packets using Wireshark and analyze the IPv4 header fields, including Source IP Address, Destination IP Address, Time-To-Live (TTL), Protocol Number, and Header Length. Explain the role of the Network layer in forwarding packets between different networks.

- Construct a local area network with multiple hosts connected through a switch in Cisco Packet Tracer. Clear the ARP cache on one of the hosts and initiate communication with another host using the ping command. Capture the communication in Wireshark and identify the ARP Request and ARP Reply packets. Analyze the sender and target MAC/IP addresses and explain how the Address Resolution Protocol resolves logical IP addresses into physical MAC addresses before data transmission.

- Develop a simple network consisting of two PCs connected through a router in Cisco Packet Tracer. Configure the required IP addresses and test connectivity using the ping command. Capture the ICMP traffic using Wireshark and identify the Echo Request and Echo Reply messages. Examine important ICMP fields such as Type, Code, Identifier, and Sequence Number, and calculate the Round Trip Time (RTT). Discuss how ICMP assists in network troubleshooting and connectivity verification.

- Create a client-server network in Cisco Packet Tracer by configuring a web server and a client PC. Enable the HTTP service on the server and access the hosted webpage from the client using a web browser. Capture the communication in Wireshark and identify the TCP three-way handshake, followed by the HTTP GET request and HTTP response messages. Analyze the application-layer communication and explain how TCP provides reliable delivery for HTTP traffic through connection establishment, acknowledgments, and ordered data transfer.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25 | Demonstrates thorough understanding and effectively integrates multiple concepts/technologies | Good understanding with proper integration of most concepts | Basic understanding; limited or partial integration | Poor understanding; unable to integrate concepts |
| Experimental Design | 30 | Well-planned, systematic implementation with optimal solution | Correct implementation with minor issues | Basic implementation with errors or inefficiencies | Incorrect or incomplete implementation |
| Use of Tools | 20 | Effective and advanced use of tools/technologies with proper configuration | Appropriate use of tools with correct execution | Limited or inconsistent use of tools | Incorrect or minimal use of tools |
| Analysis of Results | 15 | Insightful analysis with accurate interpretation and validation of results | Good analysis with reasonable interpretation | Basic analysis with limited insights | Poor or incorrect analysis of results |
| Documentation | 10 | Clear, well-structured documentation with diagrams, code, and results | Organized documentation with necessary details | Basic documentation with missing details | Poor or incomplete documentation |

# EXPERIMENT 1

## Aim

To design a simple LAN with two PCs connected through a switch, configure IPv4 addresses, verify connectivity using Ping, and analyze Ethernet frames using Wireshark.

## Software Required

- Cisco Packet Tracer

- Wireshark

## Network Topology

PC0 --------\

\

Switch (2960)

/

PC1 --------/

## IP Address Configuration

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| PC0 | 192.168.1.10 | 255.255.255.0 |
| PC1 | 192.168.1.20 | 255.255.255.0 |

Default Gateway: Not Required

## Procedure

- Open Cisco Packet Tracer.

- Place one Switch and two PCs.

- Connect using Copper Straight Through cable.

- Configure IP addresses.

- Open Command Prompt.

- Execute

ping 192.168.1.20

- Start Wireshark.

- Select Ethernet Interface.

- Capture packets.

- Stop capture after successful ping.

## Ping Output

Reply from 192.168.1.20:

Bytes=32

Time<1ms

TTL=128

Packets Sent =4

Packets Received =4

Lost =0

## Ethernet Frame Analysis

| Field | Example |
| --- | --- |
| Destination MAC | 00:1A:2B:3C:4D:5E |
| Source MAC | 00:1A:2B:3C:4D:6F |
| EtherType | 0x0800 (IPv4) |
| Payload | ICMP Data |
| FCS | CRC-32 |

## Explanation

### Source MAC Address

Physical address of sender.

### Destination MAC Address

Physical address of receiver.

### EtherType

Specifies upper-layer protocol.

0x0800 → IPv4

### Frame Check Sequence

32-bit CRC used for error detection.

## Result

Ping successful.
 Ethernet frame successfully analyzed.

# EXPERIMENT 2

## Aim

To connect two LANs using a router and analyze IPv4 packets.

## Network Topology

PC0 ----Switch0----\

Router

PC1 ----Switch0----/

|

PC2 ----Switch1----\

|

PC3 ----Switch1----/

## IP Configuration

### LAN 1

| Device | IP |
| --- | --- |
| PC0 | 192.168.1.2 |
| PC1 | 192.168.1.3 |
| Router G0/0 | 192.168.1.1 |

Subnet Mask

255.255.255.0

### LAN 2

| Device | IP |
| --- | --- |
| PC2 | 192.168.2.2 |
| PC3 | 192.168.2.3 |
| Router G0/1 | 192.168.2.1 |

## Router Configuration

enable

configure terminal

interface g0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface g0/1

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

## Ping

PC0

ping 192.168.2.2

Successful.

## IPv4 Header Analysis

| Field | Example |
| --- | --- |
| Source IP | 192.168.1.2 |
| Destination IP | 192.168.2.2 |
| TTL | 128 |
| Protocol | 1 (ICMP) |
| Header Length | 20 Bytes |

## Explanation

### TTL

Decreases by one at every router.

### Protocol Number

1 → ICMP

6 → TCP

17 → UDP

### Header Length

Normally

20 Bytes

## Role of Network Layer

- Logical addressing

- Routing

- Packet forwarding

- Path determination

## Result

Router successfully forwarded packets.

# EXPERIMENT 3

## Aim

To analyze ARP Request and Reply.

## Topology

PC0

\

Switch

/

PC1

## Procedure

- Configure IP addresses.

PC0

192.168.10.10

PC1

192.168.10.20

- Clear ARP Cache

arp -d

- Start Wireshark.

- Ping

ping 192.168.10.20

## ARP Request

Who has 192.168.10.20?

Tell 192.168.10.10

Destination MAC

FF:FF:FF:FF:FF:FF

Broadcast

## ARP Reply

192.168.10.20

is at

00:11:22:33:44:55

## Packet Analysis

| Field | Request | Reply |
| --- | --- | --- |
| Sender IP | 192.168.10.10 | 192.168.10.20 |
| Sender MAC | PC0 MAC | PC1 MAC |
| Target IP | 192.168.10.20 | 192.168.10.10 |
| Target MAC | 00:00:00:00:00:00 | PC0 MAC |

## Explanation

ARP converts

IP Address

↓

MAC Address

Before sending data, sender learns receiver MAC address.

## Result

ARP Request and Reply captured successfully.

# EXPERIMENT 4

## Aim

To capture ICMP Echo Request and Echo Reply and calculate RTT.

## Topology

PC0 ---- Router ---- PC1

## IP Configuration

PC0

192.168.1.2

Router

192.168.1.1

192.168.2.1

PC1

192.168.2.2

## Ping

ping 192.168.2.2

## Wireshark

Capture

ICMP

## ICMP Analysis

| Field | Value |
| --- | --- |
| Type | 8 |
| Code | 0 |
| Identifier | 0x0012 |
| Sequence Number | 1 |

Echo Reply

| Field | Value |
| --- | --- |
| Type | 0 |
| Code | 0 |

## RTT Calculation

Example

Request

10:10:20.100

Reply

10:10:20.104

RTT

0.004 second

=

4 ms

## ICMP Uses

- Ping

- Connectivity testing

- Route diagnosis

- Error reporting

## Result

Echo Request and Reply successfully analyzed.

# EXPERIMENT 5

## Aim

To configure an HTTP server and analyze TCP and HTTP communication.

## Topology

Client PC ---- Switch ---- Server

## IP Configuration

| Device | IP |
| --- | --- |
| Client | 192.168.1.2 |
| Server | 192.168.1.10 |

## Server Configuration

Services

↓

HTTP

↓

ON

Create webpage

Welcome to SIMATS Server

## Client

Desktop

↓

Web Browser

Enter

http://192.168.1.10

## Wireshark

Capture

tcp

## TCP Three-Way Handshake

Client

↓

SYN

↓

Server

↓

SYN ACK

↓

Client

↓

ACK

Connection Established

## HTTP

GET /

HTTP/1.1

Server Response

HTTP/1.1 200 OK

## TCP Reliability

TCP provides:

- Reliable communication

- Error detection

- Acknowledgments

- Flow control

- Ordered delivery

- Retransmission of lost packets

## Application Layer Analysis

Browser

↓

HTTP GET

↓

Server

↓

HTTP Response

↓

Webpage Displayed

## Result

HTTP communication established successfully. TCP handshake and HTTP messages were captured and analyzed.