SIMATS ENGINEERING

ASSESSMENT TOOL 3

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO4: 
Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL6)

Assessment Tool Weightage: 30%

QUESTIONS: 5								Total Marks:25

Annexure A

Comparative Analysis

- Star topology is generally more expensive than bus topology because it requires a central device such as a switch or hub and more cabling to connect each node individually. In contrast, bus topology uses a single backbone cable, making it low-cost and easier to install. However, in terms of reliability, star topology is superior because failure of one node does not affect others, whereas a failure in the main bus cable can bring down the entire network. Performance is also better in star topology since data transmission is managed through a central device, reducing collisions, while bus topology suffers from performance degradation as more devices are added due to increased data traffic on the shared medium.

- Mesh topology offers very high fault tolerance because every node is connected to multiple other nodes, providing multiple paths for data transmission. If one link fails, data can still be routed through alternate paths without disrupting communication. In contrast, star topology has moderate fault tolerance; while failure of individual nodes does not affect the network, failure of the central hub or switch can cause the entire network to stop functioning. Therefore, mesh topology is more robust and reliable in critical environments compared to star topology.

- Bus topology is simple and easy to install because it uses a single backbone cable with minimal connections, making it suitable for small networks. On the other hand, mesh topology is highly complex to install as each node must be connected to every other node, requiring a large number of cables and ports. This complexity increases significantly as the number of devices grows, making mesh topology time-consuming and costly to implement compared to the straightforward setup of bus topology.

- Hybrid topology is highly scalable because it combines two or more different topologies (such as star, mesh, or bus), allowing the network to expand flexibly according to organizational needs. New segments can be added without affecting the existing network structure. In comparison, star topology is also scalable but to a limited extent, as it depends on the capacity of the central device (switch or hub). Once the central device reaches its port limit, expansion requires additional hardware. Thus, hybrid topology provides greater scalability than star topology.

- In terms of maintenance, star topology is relatively easy to manage because issues can be quickly identified and isolated to a specific node or cable. Mesh topology, while highly reliable, is difficult to maintain due to its complex structure and large number of connections, making troubleshooting time-consuming. Bus topology is harder to maintain than star because faults in the backbone cable can disrupt the entire network and are difficult to locate. Hybrid topology has moderate maintenance complexity; while it benefits from flexibility, managing multiple combined topologies requires careful monitoring and skilled administration.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 15% | Demonstrates complete and in-depth understanding of all concepts being compared | Good understanding with minor gaps | Basic understanding; some concepts unclear | Poor understanding of concepts |
| Comparison Depth | 20% | Provides detailed, insightful, and meaningful comparisons across multiple aspects | Adequate comparison with relevant points | Limited comparison; lacks depth | Minimal or incorrect comparison |
| Criteria Selection | 10% | Selects highly relevant and appropriate comparison parameters | Mostly relevant criteria | Some irrelevant or missing criteria | Poor or no criteria selection |
| Analytical Skills | 15% | Strong analysis with logical reasoning and clear justification | Good analysis with some justification | Basic analysis with weak reasoning | No clear analysis |
| Organization & Structure | 10% | Well-organized, clear, and logically structured | Generally organized | Somewhat disorganized | Poorly structured |
| Use of Examples / Evidence | 10% | Uses strong, relevant examples or data to support comparison | Uses some examples | Limited examples | No supporting examples |
| Clarity of Presentation | 10% | Very clear, concise, and easy to understand | Mostly clear | Somewhat unclear | Difficult to understand |
| Conclusion & Insights | 10% | Insightful conclusion with strong justification | Clear conclusion | Basic conclusion | No or weak conclusion |

## Q1. Star Topology vs Bus Topology

| Criteria | Star Topology | Bus Topology |
| --- | --- | --- |
| Cost | Higher due to switch and individual cables | Lower due to single backbone cable |
| Installation | Easy but requires more cabling | Simple and requires less cabling |
| Reliability | High; failure of one node does not affect others | Low; backbone failure affects entire network |
| Performance | Better performance and fewer collisions | Performance decreases as traffic increases |
| Maintenance | Easy to identify individual faults | Difficult to locate backbone faults |
| Scalability | Good, depending on switch ports | Limited |
| Example | Modern office LAN | Small/legacy classroom network |

Conclusion: Star topology is more expensive but provides better performance, reliability, maintenance and scalability. Bus topology is preferable when low cost and simple installation are the main requirements.

# Q2. Mesh Topology vs Star Topology – Fault Tolerance

| Criteria | Mesh Topology | Star Topology |
| --- | --- | --- |
| Fault tolerance | Very high | Moderate |
| Alternative paths | Multiple paths available | Usually no alternative path through another switch |
| Link failure | Traffic can use another path | Affected node loses connection |
| Central device failure | No single central dependency in full mesh | Entire network can be affected |
| Reliability | Extremely high | High |
| Cost | Very expensive | Relatively economical |
| Example | Banking/core networks | Office LAN |

Conclusion: Mesh topology provides superior fault tolerance and reliability because multiple paths are available. Star topology is simpler and cheaper but depends heavily on the central switch.

# Q3. Bus Topology vs Mesh Topology – Complexity

| Criteria | Bus Topology | Mesh Topology |
| --- | --- | --- |
| Structure | Single backbone | Multiple interconnected links |
| Installation | Simple | Complex |
| Cabling | Low | Very high |
| Number of connections | Few | Many |
| Cost | Low | Very high |
| Troubleshooting | Backbone faults can be difficult | Complex because of many connections |
| Scalability | Limited | Difficult and expensive to scale |
| Example | Small lab | Critical network backbone |

### Technical Point

For a full mesh with n devices, the number of links required is:

Links = n(n − 1) / 2

For example, with 5 devices:

5(5 − 1) / 2 = 10 links

Therefore, mesh complexity increases rapidly as more devices are added.

Conclusion: Bus topology is much simpler and cheaper to implement, while mesh topology provides better reliability at the cost of greater complexity and expense.

# Q4. Hybrid Topology vs Star Topology – Scalability

| Criteria | Hybrid Topology | Star Topology |
| --- | --- | --- |
| Scalability | Very high | Good |
| Expansion | New network segments can be added | Limited by central switch capacity |
| Flexibility | Very high | Moderate |
| Network size | Suitable for large networks | Suitable for small/medium networks |
| Cost | Higher | Lower |
| Management | More complex | Easier |
| Example | University campus | Office LAN |

Conclusion: Hybrid topology provides greater scalability and flexibility because different topologies can be combined according to requirements. Star topology is easier to manage but expansion depends on the capacity of the central switch.

# Q5. Maintenance Comparison of Network Topologies

| Criteria | Star | Mesh | Bus | Hybrid |
| --- | --- | --- | --- | --- |
| Maintenance | Easy | Difficult | Moderate/Difficult | Moderate |
| Fault detection | Easy | Complex | Difficult | Moderate |
| Troubleshooting | Simple | Time-consuming | Backbone faults are difficult | Depends on topology |
| Reliability | High | Very high | Low | High |
| Administration | Easy | Complex | Simple initially | Requires skilled administration |
| Cost of maintenance | Low/Moderate | High | Low initially | Moderate/High |

### Analysis

- Star: Individual cable or device failures can be easily identified.

- Mesh: Highly reliable but has many links, making troubleshooting complicated.

- Bus: Backbone failure can affect all devices and locating the fault can be difficult.

- Hybrid: Maintenance depends on the combination of topologies used.

Conclusion: Star topology is generally the easiest to maintain, while mesh requires the most complex maintenance. Hybrid topology provides flexibility but requires proper network planning and administration.

# Overall Comparative Analysis

| Parameter | Star | Mesh | Bus | Hybrid |
| --- | --- | --- | --- | --- |
| Cost | Medium | Very High | Low | High |
| Reliability | High | Very High | Low | High |
| Performance | High | Very High | Low/Medium | High |
| Scalability | Good | Poor/Complex | Low | Very High |
| Maintenance | Easy | Difficult | Difficult | Moderate |
| Fault Tolerance | Medium | Excellent | Poor | High |
| Complexity | Low | Very High | Low | High |
| Best Application | Offices | Banks/Data Centers | Small Labs | Universities/Enterprises |

## Final Conclusion

Each topology has different strengths. Bus topology is suitable for low-cost, small networks, while star topology provides better performance and easier maintenance. Mesh topology is ideal for critical networks requiring maximum reliability and fault tolerance. Hybrid topology is best for large organizations because it combines multiple topologies to achieve scalability, flexibility, reliability and efficient network management.