SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO4: 
Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL6)

Assessment Tool Weightage: 30%

Annexure A

Industry Problem-Solving Task

- A star topology is ideal for a 20-person startup because all devices connect to a central switch, making the network easy to manage and troubleshoot. At the core sits a Layer 2/Layer 3 managed switch (such as a Cisco Catalyst or Netgear ProSafe) with at least 24 ports to accommodate all workstations, printers, and VoIP phones with room to grow. Each device connects to the switch using Cat6 UTP cables, which support Gigabit speeds up to 100 meters — more than sufficient for any typical office floor plan. The switch uplinks to a router/firewall (such as a Cisco RV series or pfSense appliance) that handles internet access, DHCP, and network address translation. A Wi-Fi access point connected to the switch extends wireless coverage for laptops and mobile devices. The key advantage of this design is fault isolation: if one workstation's cable fails, only that single node goes offline, leaving all other 19 employees unaffected. Centralised cable management also simplifies future moves, adds, and changes.

- A bank demands near-zero downtime, making a full or partial mesh topology the right choice. In a full mesh, every core network device — routers, core switches, and servers — connects directly to every other device, so no single link or node failure can bring down the network. For a bank, a partial mesh is typically deployed at the core layer (connecting all core switches and firewalls to each other) while the distribution and access layers use redundant uplinks to at least two core switches. Dual enterprise-grade routers (such as Cisco ASR or Juniper MX series) provide BGP-based redundant internet connectivity. All links between core devices use fiber optic cables — either single-mode for long distances between buildings or multi-mode OM4 within the data center — because fiber is immune to electromagnetic interference, supports very high bandwidth, and is essential for maintaining low-latency financial transactions. Redundant power supplies, UPS systems, and RAID storage arrays further complement the mesh design. The justification is straightforward: when a link fails in a mesh, traffic automatically reroutes through alternative paths in milliseconds using protocols like OSPF or BGP, satisfying regulatory requirements for uptime and data integrity.

- For a small classroom lab with a tight budget, bus topology offers the lowest hardware cost because all computers share a single common communication line — the bus — eliminating the need for a central switch or hub. A single coaxial cable (RG-58, 50-ohm) or, in a modern variation, a single Cat5e/Cat6 cable daisy-chained through an inexpensive unmanaged hub runs along the length of the room. Each computer connects to the backbone using a T-connector and a BNC connector in a classic coaxial setup, with terminators (50-ohm) placed at both ends of the cable to absorb signals and prevent reflection. For a more contemporary low-cost implementation, a single inexpensive 8- or 16-port unmanaged switch replaces the terminators and uses Cat5e UTP patch cables to each workstation — achieving bus-like simplicity at minimal cost. The primary limitation is that the network is susceptible to collisions and a single cable break disrupts all connected machines, but for a supervised classroom environment with light usage (internet browsing, file sharing), this trade-off is entirely acceptable. No dedicated server is required; a teacher's PC can share resources using built-in Windows/Linux file sharing.

- A large university campus serves thousands of users across multiple buildings — lecture halls, labs, libraries, dormitories, and administration — making a single topology inadequate. A hybrid topology combines the best elements of multiple designs: a fiber-based ring or mesh at the campus backbone level for redundancy, a star topology within each building for manageability, and bus or tree structures within individual labs for cost-efficiency. At the campus core, high-speed fiber optic cables (single-mode, 10 Gbps or 40 Gbps) interconnect core routers and multilayer switches using a ring or partial mesh arrangement, ensuring that no single link failure isolates an entire building. Each building has its own distribution switch connected to the campus core via dual fiber uplinks. Within each building, a traditional star topology branches from the distribution switch to floor-level access switches, which then connect individual devices via Cat6 cables. This hierarchical three-layer model — core, distribution, and access — is the industry standard for enterprise and campus networks because adding a new building simply means installing a new distribution switch and running fiber to the core, with no disruption to existing infrastructure. Wireless access points throughout the campus connect via PoE (Power over Ethernet) to access switches, eliminating the need for separate power cabling. The hybrid design thus delivers redundancy at the backbone, simplicity at the building level, and scalability campus-wide.

- Designing a network for a company with multiple departments involves creating a structured and scalable architecture using switches and routers to ensure efficient communication and security. Each department (such as HR, Finance, Sales, and IT) is connected through dedicated Layer 2 switches that form the access layer, allowing devices like computers and printers to communicate within the same department. These switches are then connected to a central Layer 3 switch or router, which acts as the core layer and enables communication between different departments through inter-VLAN routing. Virtual LANs (VLANs) are configured to logically separate each department’s network, improving security and reducing broadcast traffic. A router is also used to connect the internal network to external networks such as the internet, often with firewall functionality for security. Redundant links and backup paths can be included to ensure reliability, while proper IP addressing and subnetting are applied for efficient network management. This hierarchical design improves performance, enhances security, and allows easy expansion as the company grows.

## 1. 20-Person Startup – Star Topology

### Problem Analysis

A startup with 20 employees needs a network that is easy to manage, troubleshoot, and expand. A star topology is suitable because every device connects to a central switch.

### Proposed Network Design

- 24-port managed switch as the central device

- Router/Firewall for Internet connectivity

- Cat6 UTP cables for wired connections

- Wi-Fi Access Point for wireless devices

- Computers, printers and VoIP phones connected to the switch

- DHCP used for automatic IP allocation

### Network Diagram

INTERNET

|

Router/Firewall

|

24-Port Switch

_____________|_____________

|       |       |            |

PC1     PC2     PC3         PC20

|       |       |            |

Cat6    Cat6    Cat6         Cat6

|

Wi-Fi AP

/    |    \

Laptop Mobile Printer

### Advantages

- Easy installation and management

- Failure of one cable affects only one device

- Easy troubleshooting

- Simple to expand

- Good performance

### Conclusion

Star topology is the best choice for the startup because it provides centralized management, easy fault isolation and future scalability.

# 2. Bank – Mesh Topology

### Problem Analysis

A bank requires high availability and minimum downtime because network failure can affect financial transactions.

### Proposed Network Design

A partial mesh topology is recommended at the core layer.

### Network Components

- Two enterprise routers

- Two or more core Layer 3 switches

- Firewalls

- Servers

- Fiber-optic cables

- OSPF/BGP for routing

- UPS and redundant power supplies

### Network Diagram

Internet 1

|

Router 1

/    \

/      \

Core SW1----Core SW2

|\        /|

| \      / |

|  \    /  |

Server 1  Server 2

|

Firewall

|

Department LANs

### Advantages

- High reliability

- Multiple alternative paths

- Very low downtime

- Automatic rerouting

- Suitable for critical banking applications

### Conclusion

A partial mesh topology provides the bank with redundancy, reliability and continuous network availability.

# 3. Classroom Lab – Bus Topology

### Problem Analysis

A small classroom with limited budget requires a low-cost network. A traditional bus topology uses a common backbone cable.

### Components

- Coaxial RG-58 cable

- BNC connectors

- T-connectors

- 50-ohm terminators

- Computers

- Network adapters

### Network Diagram

50Ω Terminator

|

|

------|---------|---------|---------|------

|         |         |         |

PC1       PC2       PC3       PC4

|         |         |         |

T-Connector T-Connector T-Connector

|

50Ω Terminator

### Advantages

- Low hardware cost

- Simple design

- Requires less cable

- Suitable for small networks

### Limitations

- Backbone failure affects the entire network

- Collision possibility

- Difficult troubleshooting

- Limited scalability

### Conclusion

Bus topology can be used for a small, low-traffic classroom lab where cost is more important than high performance.

# 4. University Campus – Hybrid Topology

### Problem Analysis

A university has thousands of users distributed across multiple buildings. A single topology cannot efficiently handle such a large network.

### Proposed Design

A hybrid topology combines:

- Mesh/ring → Campus backbone

- Star → Individual buildings

- Tree → Floor-level network

### Network Architecture

CAMPUS INTERNET

|

Core Router

/         \

Core Switch 1---Core Switch 2

/   \           /   \

/     \         /     \

Building A  Building B  Building C

|          |           |

Distribution  Distribution Distribution

Switch       Switch       Switch

|            |            |

Access        Access       Access

Switches      Switches     Switches

/ | \         / | \        / | \

PCs APs PCs    PCs APs PCs  PCs APs PCs

### Transmission Media

- Single-mode fiber → Long-distance campus connections

- Multi-mode fiber → Building/data-center connections

- Cat6 UTP → Individual devices

- PoE → Wireless access points

### Advantages

- Highly scalable

- Reliable backbone

- Easy building expansion

- Good performance

- Centralized management

### Conclusion

A hybrid topology is the best choice for a university because it combines reliability, scalability and cost efficiency.

# 5. Multi-Department Company – VLAN-Based Network

### Problem Analysis

Departments such as HR, Finance, Sales and IT need to communicate while maintaining network security and separation.

### Proposed Design

A hierarchical network using VLANs, Layer 2 switches and a Layer 3 switch/router is suitable.

### VLAN Structure

| Department | VLAN | Example Network |
| --- | --- | --- |
| HR | VLAN 10 | 192.168.10.0/24 |
| Finance | VLAN 20 | 192.168.20.0/24 |
| Sales | VLAN 30 | 192.168.30.0/24 |
| IT | VLAN 40 | 192.168.40.0/24 |

### Network Diagram

INTERNET

|

Router / Firewall

|

Layer 3 Switch

_________|_________

|         |         |

VLAN 10   VLAN 20   VLAN 30

HR      Finance    Sales

|         |         |

L2 SW     L2 SW     L2 SW

/|\       /|\       /|\

PCs PCs    PCs PCs    PCs PCs

|

VLAN 40

IT

|

L2 SW

### Technical Implementation

- Create separate VLANs for each department

- Configure access ports on Layer 2 switches

- Configure trunk links between switches

- Configure inter-VLAN routing on the Layer 3 switch

- Use DHCP for automatic IP assignment

- Configure firewall rules for security

- Apply subnetting for efficient IP management

### Advantages

- Improved security

- Reduced broadcast traffic

- Better network management

- Departmental isolation

- Easy expansion

- Efficient communication

### Conclusion

A VLAN-based hierarchical network provides security, performance and scalability for a multi-department company.

# Overall Testing and Validation

The proposed networks can be tested using Cisco Packet Tracer.

### Testing Steps

- Connect all devices according to the topology.

- Assign IP addresses.

- Configure switches and routers.

- Configure VLANs where required.

- Test connectivity using ping.

- Check routing between networks.

- Disconnect a link and verify redundancy where applicable.

- Verify that unaffected devices continue communicating.

### Expected Result

| Scenario | Recommended Topology | Main Benefit |
| --- | --- | --- |
| Startup | Star | Easy management |
| Bank | Mesh | High reliability |
| Classroom | Bus | Low cost |
| University | Hybrid | Scalability |
| Company | VLAN + Hierarchical | Security & management |

## Final Conclusion

The correct topology depends on the size, cost, reliability, scalability and security requirements of the organization. Star topology is suitable for small offices, mesh provides high availability for banks, bus can be used for low-cost classroom networks, hybrid topology is ideal for large campuses, and VLAN-based hierarchical design provides secure and efficient communication between company departments.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Problem Analysis | 15% | Clearly identifies and deeply analyzes the industry problem with all factors | Good understanding with minor gaps | Basic understanding; some key aspects missing | Poor or incorrect understanding of the problem |
| Solution Design | 20% | Innovative, practical, and industry-relevant solution | Effective solution with minor limitations | Partially suitable solution | Inappropriate or unclear solution |
| Technical Implementation | 20% | Fully functional, accurate, and well-executed implementation | Mostly functional with minor errors | Partially implemented solution | Incomplete or non-functional implementation |
| Use of Tools & Technologies | 10% | Appropriate and advanced use of relevant tools and technologies | Correct use of tools | Limited or basic use of tools | Incorrect or no use of tools |
| Problem-Solving Approach | 10% | Logical, systematic, and well-structured approach | Mostly logical approach | Somewhat disorganized approach | No clear approach |
| Innovation & Creativity | 10% | Highly creative and original solution | Some creativity | Limited originality | No creativity |
| Testing & Validation | 5% | Thorough testing with real-world scenarios | Adequate testing | Minimal testing | No testing |
| Documentation & Presentation | 5% | Clear, professional, and well-documented | Good documentation | Basic documentation | Poor or no documentation |
| Teamwork / Collaboration | 5% | Excellent coordination and contribution (if team-based) | Good collaboration | Limited participation | No collaboration or contribution |