SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO5: 
Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL5)

Assessment Tool Weightage: 40%

QUESTIONS: 3								Total Marks:30

Annexure A

Constraint-Based Problem Solving

- A  multinational company is opening a new branch office in a remote rural location where high-speed wired internet is unavailable. The office must support cloud-based applications, video conferencing, and secure communication with the headquarters. The company has a limited budget for connectivity but requires uninterrupted business operations. As a network engineer, recommend a suitable internet connection, networking devices, and security mechanisms. Justify your recommendations by considering bandwidth limitations, reliability, cost, and future scalability

- A university plans to provide campus-wide wireless internet access for more than 3,000 students across classrooms, laboratories, libraries, and hostels. The network must support a large number of simultaneous users while minimizing signal interference and ensuring secure access. Due to budget constraints, only a limited number of wireless access points can be deployed. Analyze the requirements and propose an efficient wireless network design, including access point placement, frequency band selection, and authentication methods. Explain how your solution balances coverage, performance, security, and cost.

- A financial institution is upgrading its internal computer network to improve cybersecurity while allowing employees to access sensitive customer information and online banking services. The organization must comply with strict security policies without significantly affecting network performance or increasing operational costs. Design a secure network architecture by selecting appropriate network segmentation techniques, firewall deployment, authentication methods, and intrusion detection mechanisms. Justify your design by considering the constraints of security, performance, compliance, and budget.

RUBRICS FOR CALCULATION

| Criteria | Weightage (%) | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Problem & Constraints | 20% | Clearly identifies all constraints and fully understands the problem requirements | Identifies most constraints with minor gaps in understanding | Identifies some constraints but misses key aspects | Shows little or no understanding of constraints or problem |
| Problem Decomposition | 15% | Breaks problem into logical, well-structured sub-problems effectively | Breaks problem into sub-parts with some logical flow | Attempts decomposition but lacks clarity or structure | No clear decomposition of the problem |
| Application of Constraints in Solution | 20% | Applies all constraints correctly and consistently in solution design | Applies most constraints correctly with minor errors | Applies some constraints but with noticeable errors | Fails to apply constraints appropriately |
| Logical Reasoning & Strategy | 15% | Uses strong, clear, and efficient reasoning strategies | Uses generally sound reasoning with minor gaps | Reasoning is partially correct but inconsistent | Reasoning is unclear or incorrect |
| Solution Accuracy & Feasibility | 15% | Solution is fully correct, feasible, and optimized | Solution is mostly correct and feasible | Solution has errors or is partially feasible | Solution is incorrect or not feasible |
| Creativity & Innovation | 5% | Demonstrates highly innovative and unique approach | Shows some creativity in approach | Limited creativity shown | No creativity; relies on basic or copied ideas |
| Clarity of Presentation | 10% | Solution is clearly presented with excellent organization and explanation | Presentation is clear with minor issues | Presentation lacks clarity or organization | Poorly presented and difficult to understand |

# QUESTION 1 – Remote Rural Branch Office

### 1. Problem Analysis

The company needs a reliable connection for cloud applications, video conferencing and secure communication with headquarters.

### Main Constraints

- High-speed wired Internet is unavailable.

- Limited connectivity budget.

- Cloud applications require stable bandwidth.

- Video conferencing requires low latency and consistent connectivity.

- Secure communication with headquarters is essential.

- Business operations should continue during connection failures.

- The solution should support future expansion.

### 2. Proposed Solution

A 4G/5G wireless Internet connection is recommended as the primary connection. If 5G coverage is unavailable, 4G LTE can be used.

A dual-WAN router can be used with a secondary connection such as another 4G/5G provider or satellite Internet as backup.

### Network Architecture

4G/5G Internet

|

Dual-WAN Router

|

Firewall/VPN

|

Managed Switch

_________|_________

|         |         |

PCs      Printer    Wi-Fi AP

|

Laptops/Mobiles

|

Secure VPN Tunnel

|

Headquarters

### 3. Required Devices

| Device | Purpose |
| --- | --- |
| 4G/5G Modem | Provides Internet connectivity |
| Dual-WAN Router | Manages primary and backup connections |
| Firewall | Protects the internal network |
| Managed Switch | Connects office devices |
| Wi-Fi Access Point | Provides wireless access |
| VPN Gateway | Securely connects branch to headquarters |

### 4. Security Mechanisms

- IPsec VPN for secure branch-to-HQ communication

- Firewall rules to block unauthorized traffic

- WPA3 for Wi-Fi security

- Strong administrator passwords

- Network segmentation using VLANs

- Regular firmware/software updates

### 5. Constraint-Based Justification

Bandwidth: 4G/5G can provide sufficient bandwidth for cloud applications and video conferencing where coverage permits.

Reliability: Dual-WAN provides a backup connection if the primary connection fails.

Cost: Using cellular connectivity avoids expensive wired infrastructure deployment in a rural location.

Security: VPN encryption protects communication between the branch and headquarters.

Scalability: A managed switch and dual-WAN router allow additional users and devices to be added later.

### Conclusion

A 4G/5G-based dual-WAN network with firewall and VPN provides a practical balance between cost, reliability, security, bandwidth and scalability.

# QUESTION 2 – University Wireless Network

## 1. Problem Analysis

The university needs wireless connectivity for more than 3,000 students across classrooms, laboratories, libraries and hostels.

### Main Constraints

- More than 3,000 simultaneous users

- Large geographical coverage

- Limited number of access points

- High user density

- Signal interference must be minimized

- Secure authentication is required

- Budget is limited

## 2. Proposed Wireless Design

A centrally managed Wi-Fi 6/6E network is recommended.

Instead of simply increasing the number of APs, access points should be strategically placed in high-density areas.

### Network Architecture

Internet

|

Core Router

|

Layer 3 Switch

|

Wireless LAN Controller

_______|_______

|       |       |

AP1     AP2     AP3

|       |       |

Classrooms  Labs   Library

|

Hostels

### 3. Access Point Placement

| Area | AP Strategy |
| --- | --- |
| Classrooms | APs placed centrally |
| Laboratories | APs positioned based on user density |
| Library | Higher-density AP deployment |
| Hostels | APs distributed floor-wise |
| Outdoor areas | Outdoor-rated APs where required |

Access points should be positioned after a Wi-Fi site survey to identify dead zones and interference.

### 4. Frequency Band Selection

2.4 GHz

- Longer range

- Better wall penetration

- More interference

- Limited non-overlapping channels

5 GHz

- Higher performance

- More channels

- Less interference than 2.4 GHz

- Suitable for most student devices

6 GHz

- More spectrum and less congestion where supported

- Suitable for newer Wi-Fi 6E/7-compatible devices

### Recommended Approach

Use 5 GHz as the primary band, with 2.4 GHz for legacy/low-bandwidth devices. Use 6 GHz where compatible devices and local regulatory requirements permit.

### 5. Authentication and Security

Use:

- WPA3-Enterprise

- 802.1X authentication

- RADIUS server

- University student/staff credentials

- Separate guest network

- VLAN-based network segmentation

### 6. Managing Interference

- Conduct wireless site surveys

- Use automatic channel selection

- Reduce unnecessary AP transmission power

- Use appropriate channel widths

- Avoid overlapping channels

- Use band steering to move capable devices toward 5 GHz/6 GHz

### 7. Balancing the Constraints

Coverage: Strategic AP placement covers classrooms, labs, libraries and hostels.

Performance: 5 GHz/6 GHz and Wi-Fi 6 technologies support high-density environments.

Security: WPA3-Enterprise and 802.1X provide strong user authentication.

Cost: APs are deployed according to user density instead of placing excessive APs everywhere.

### Conclusion

A centrally managed Wi-Fi 6/6E network with strategic AP placement, 5 GHz prioritization, WPA3-Enterprise and 802.1X authentication provides a good balance of coverage, performance, security and cost.

# QUESTION 3 – Secure Financial Institution Network

## 1. Problem Analysis

A financial institution handles sensitive customer information and online banking services. Therefore, the network must provide strong security without significantly reducing performance or increasing operational costs.

### Main Constraints

- Sensitive customer data

- Strict cybersecurity requirements

- Compliance requirements

- Employees need controlled access

- Online banking services must remain available

- High network performance is required

- Budget and operational costs must be controlled

# 2. Proposed Network Architecture

A segmented, firewall-protected, multi-layer architecture is recommended.

### Network Diagram

INTERNET

|

Perimeter Firewall

|

DMZ Network

_________|_________

|                   |

Web Server          Online Banking

|

Internal Firewall

|

Core Layer-3 Switch

_______|_______|________

|       |       |         |

HR VLAN Finance  IT VLAN  User VLAN

VLAN

|

Customer Database

|

IDS/IPS System

### 3. Network Segmentation

Use VLANs to separate departments and sensitive systems.

| Network | Purpose |
| --- | --- |
| VLAN 10 | HR |
| VLAN 20 | Finance |
| VLAN 30 | IT |
| VLAN 40 | General Employees |
| VLAN 50 | Servers |
| VLAN 60 | Guest Network |
| DMZ | Public-facing services |

Sensitive servers should be isolated from normal employee networks.

### 4. Firewall Deployment

Use firewalls at multiple security boundaries:

Internet → Perimeter Firewall → DMZ → Internal Firewall → Internal Network

Firewall rules should allow only required services and block unauthorized connections.

### 5. Authentication

Use:

- Multi-factor authentication (MFA)

- Strong passwords

- Role-Based Access Control (RBAC)

- 802.1X for network access

- RADIUS/AAA authentication

- Least-privilege access

### 6. Intrusion Detection and Prevention

Deploy an IDS/IPS to monitor suspicious network activity.

It can detect:

- Port scanning

- Unauthorized access

- Malware traffic

- Abnormal network behavior

- Suspicious login attempts

An IPS can automatically block detected malicious traffic.

### 7. Performance and Cost Optimization

- Use Layer 3 switches for high-speed internal routing.

- Apply VLANs to reduce unnecessary broadcast traffic.

- Use firewall policies based on business requirements.

- Centralize network monitoring.

- Use existing enterprise infrastructure where possible.

- Prioritize security controls for critical systems.

### 8. Constraint-Based Justification

Security: VLANs, firewalls, MFA and IDS/IPS protect sensitive information.

Performance: Layer 3 switching provides fast internal communication and reduces unnecessary traffic.

Compliance: Authentication, access control and monitoring support security-policy requirements.

Budget: Network segmentation and software-based security controls can be implemented without completely replacing existing infrastructure.

### Conclusion

A VLAN-segmented network with layered firewalls, MFA, RBAC and IDS/IPS provides strong protection for a financial institution while maintaining performance, compliance, availability and cost efficiency.

# Overall Comparison of Solutions

| Problem | Recommended Solution | Main Constraint Addressed |
| --- | --- | --- |
| Rural Branch | 4G/5G + Dual-WAN + VPN | Limited wired connectivity |
| University | Wi-Fi 6/6E + Strategic APs | 3,000+ users with limited APs |
| Financial Institution | VLAN + Firewall + MFA + IDS/IPS | Security and compliance |

## Final Conclusion

The three problems require different network strategies because their constraints are different. The rural branch requires reliable wireless connectivity and VPN security, the university requires high-density wireless optimization, and the financial institution requires strong network segmentation and layered security. In each case, the proposed design balances performance, reliability, security, scalability and cost, making the solutions practical for real-world network environments.