SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO5: Measure network performance using key metrics with simulation tools. (BL4,BL5)

Assessment Tool Weightage: 30%

Annexure A

Pseudocode Writing Task (Algorithm Design)

- In a network simulation using Cisco Packet Tracer, a student records total data received as 8000 bits in 4 seconds and observes that 200 packets were sent while 180 packets were received. Write a pseudocode algorithm to calculate the network throughput in bits per second and the packet loss percentage based on the given values. The algorithm should include validation to ensure that time and total packets sent are not zero to avoid errors. It should display both the throughput and packet loss results clearly. Add logic to classify the network as Efficient if throughput is greater than 1500 bps and packet loss is less than 10%, otherwise classify it as Inefficient. Finally, ensure the algorithm outputs a complete summary of the network performance based on the calculated metrics.

- An organization has multiple network links connecting its branches. The administrator wants to monitor bandwidth utilization and identify overloaded links. Write a pseudocode algorithm that periodically collects the bandwidth usage of each network link and stores the values in an array or list. Use a loop to continuously monitor all links. Add conditions to detect when bandwidth utilization exceeds 80% and recommend switching traffic to a backup link. Explain how this algorithm supports efficient bandwidth management and prevents network congestion.

- A company has six branch offices connected through multiple network links. Each link has different bandwidth, delay, and utilization values, which change throughout the day. The network administrator wants to continuously identify the best communication path between the headquarters and each branch based on overall link quality rather than just the shortest distance.Write a pseudocode algorithm that periodically collects the bandwidth, delay, and utilization of every network link and stores the values in suitable arrays or lists. Use a loop to repeat the monitoring process at regular intervals. Calculate a link quality score for each path using the collected metrics and select the path with the highest score. Include conditions to detect when a link becomes overloaded or fails, automatically selecting an alternate path and generating an alert for the administrator. Explain how your algorithm improves network reliability, routing efficiency, and fault tolerance while adapting to changing network conditions.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Identifies all inputs, outputs, constraints accurately | Minor missing elements | Partial understanding | Incorrect interpretation |
| Algorithm Design | 30% | Complete, correct, and logically sound solution | Minor logical gaps | Partially correct logic | Incorrect approach |
| Use of Control Structures | 20% | Efficient and appropriate use of loops, conditions, functions/modules | Minor inefficiencies | Limited or basic usage | Incorrect or missing constructs |
| Pseudocode Structure | 10% | Well-structured, clear, consistent format, easy to follow | Mostly clear with minor issues | Difficult to follow in parts | Poorly structured and unclear |
| Completeness | 20% | Handles all cases; optimized approach | Handles most cases; reasonable efficiency | Handles basic cases only | Incomplete or inefficient |

# QUESTION 1 – Throughput and Packet Loss

### Problem Understanding

Given:

- Total data received = 8000 bits

- Time = 4 seconds

- Packets sent = 200

- Packets received = 180

### Formulas

Throughput:

Throughput = Total Data Received / Time

Packet Loss:

Packet Loss % = ((Packets Sent - Packets Received) / Packets Sent) × 100

### Pseudocode

ALGORITHM Network_Performance

BEGIN

INPUT totalDataReceived

INPUT time

INPUT packetsSent

INPUT packetsReceived

IF time <= 0 THEN

DISPLAY "Error: Time must be greater than zero"

STOP

END IF

IF packetsSent <= 0 THEN

DISPLAY "Error: Total packets sent must be greater than zero"

STOP

END IF

IF packetsReceived < 0 OR packetsReceived > packetsSent THEN

DISPLAY "Error: Invalid packet values"

STOP

END IF

throughput ← totalDataReceived / time

lostPackets ← packetsSent - packetsReceived

packetLoss ← (lostPackets / packetsSent) × 100

DISPLAY "----- Network Performance Summary -----"

DISPLAY "Throughput = ", throughput, " bps"

DISPLAY "Packets Sent = ", packetsSent

DISPLAY "Packets Received = ", packetsReceived

DISPLAY "Packets Lost = ", lostPackets

DISPLAY "Packet Loss = ", packetLoss, "%"

IF throughput > 1500 AND packetLoss < 10 THEN

DISPLAY "Network Status: EFFICIENT"

ELSE

DISPLAY "Network Status: INEFFICIENT"

END IF

END

### Expected Result

Throughput:

8000 / 4 = 2000 bps

Packet Loss:

((200 - 180) / 200) × 100 = 10%

Since throughput is greater than 1500 bps but packet loss is not less than 10%, the network is:

Network Status: INEFFICIENT

# QUESTION 2 – Bandwidth Utilization Monitoring

### Problem Understanding

The organization has multiple network links. The algorithm must:

- Monitor all links continuously.

- Store bandwidth utilization values.

- Detect utilization above 80%.

- Recommend backup traffic routing.

- Repeat monitoring periodically.

### Pseudocode

ALGORITHM Bandwidth_Monitoring

BEGIN

INPUT numberOfLinks

IF numberOfLinks <= 0 THEN

DISPLAY "Error: Number of links must be greater than zero"

STOP

END IF

CREATE utilization[numberOfLinks]

WHILE TRUE DO

DISPLAY "----- Bandwidth Monitoring -----"

FOR i ← 0 TO numberOfLinks - 1 DO

INPUT utilization[i]

IF utilization[i] < 0 OR utilization[i] > 100 THEN

DISPLAY "Invalid utilization for Link ", i + 1

CONTINUE

END IF

DISPLAY "Link ", i + 1,

" Utilization = ", utilization[i], "%"

IF utilization[i] > 80 THEN

DISPLAY "WARNING: Link ", i + 1, " is overloaded"

DISPLAY "Recommendation: Switch traffic to backup link"

ELSE

DISPLAY "Link ", i + 1, " is operating normally"

END IF

END FOR

WAIT monitoringInterval

END WHILE

END

### How It Helps

- Continuously monitors network links.

- Detects overloaded links early.

- Prevents excessive congestion.

- Allows traffic to be redirected to backup links.

- Improves bandwidth utilization.

- Helps maintain better network performance.

### Example

Link 1 → 65% → Normal

Link 2 → 72% → Normal

Link 3 → 87% → Overloaded

Link 4 → 55% → Normal

Recommendation:

Switch traffic from Link 3 to backup link.

# QUESTION 3 – Best Path Selection Based on Link Quality

### Problem Understanding

The company has six branches and multiple network paths. The best path should be selected using:

- Bandwidth

- Delay

- Utilization

- Link availability

The algorithm must continuously monitor these values and select the path with the highest quality score.

## Link Quality Score

A simple scoring formula can be used:

Score = (Bandwidth × 0.5)

+ ((100 - Delay) × 0.2)

+ ((100 - Utilization) × 0.3)

Higher score = Better path.

Bandwidth, delay and utilization should be normalized to suitable comparable ranges before applying the formula in an actual implementation.

## Pseudocode

ALGORITHM Best_Path_Selection

score[i] ← 0

CONTINUE

END IF

IF status[i] = "FAILED" THEN

score[i] ← 0

CONTINUE

END IF

IF utilization[i] < 0 OR utilization[i] > 100 THEN

DISPLAY "Invalid utilization for Link ", i + 1

score[i] ← 0

CONTINUE

END IF

IF utilization[i] > 80 THEN

DISPLAY "WARNING: Link ", i + 1, " is overloaded"

END IF

score[i] ←

(bandwidth[i] × 0.5)

+ ((100 - delay[i]) × 0.2)

+ ((100 - utilization[i]) × 0.3)

DISPLAY "Link ", i + 1,

" Quality Score = ", score[i]

END FOR

bestScore ← -1

bestLink ← -1

FOR i ← 0 TO numberOfLinks - 1 DO

IF score[i] > bestScore THEN

bestScore ← score[i]

bestLink ← i

END IF

END FOR

IF bestLink = -1 THEN

DISPLAY "ERROR: No available network path"

DISPLAY "ALERT: Administrator notification required"

ELSE

DISPLAY "Best Link = Link ", bestLink + 1

DISPLAY "Best Quality Score = ", bestScore

END IF

FOR i ← 0 TO numberOfLinks - 1 DO

IF status[i] = "FAILED" THEN

DISPLAY "ALERT: Link ", i + 1, " has failed"

END IF

IF utilization[i] > 80 THEN

DISPLAY "ALERT: Link ", i + 1, " is overloaded"

END IF

END FOR

WAIT monitoringInterval

END WHILE

END

## Example

Suppose the administrator monitors three possible links:

| Link | Bandwidth | Delay | Utilization | Status |
| --- | --- | --- | --- | --- |
| Link 1 | 100 Mbps | 30 ms | 50% | Active |
| Link 2 | 80 Mbps | 20 ms | 40% | Active |
| Link 3 | 120 Mbps | 70 ms | 90% | Active |

The algorithm calculates the quality score of each link and selects the link with the highest score.

If the selected link later:

- Fails → select another available link.

- Exceeds 80% utilization → recommend an alternate path.

- Experiences high delay → its quality score decreases.

## Benefits of the Algorithm

### 1. Network Reliability

Continuously monitors network conditions and identifies failures.

### 2. Routing Efficiency

Selects paths based on multiple performance parameters rather than distance alone.

### 3. Fault Tolerance

Automatically chooses an alternate path when the preferred link fails.

### 4. Congestion Prevention

Detects links exceeding 80% utilization and recommends traffic redirection.

### 5. Adaptive Network Management

Because measurements are collected periodically, the selected path can change according to current network conditions.

# Overall Summary

| Question | Main Metric/Function | Main Output |
| --- | --- | --- |
| Q1 | Throughput + Packet Loss | Network efficiency |
| Q2 | Bandwidth Utilization | Overloaded link detection |
| Q3 | Bandwidth + Delay + Utilization | Best available path |

### Final Conclusion

These pseudocode algorithms provide a systematic method for measuring and improving network performance. Q1 calculates throughput and packet loss, Q2 monitors bandwidth utilization and detects congestion, and Q3 dynamically selects the best network path based on changing link conditions. Together, they support efficient routing, reliability, congestion control and fault tolerance in a simulated network environment such as Cisco Packet Tracer.